EE 316

Section 3

Pre Lab 11

By: Hannah Kelley

MOSFETs

Introduction:

This lab investigates the operating characteristics of an n-channel MOSFET by examining the relationship between drain current, drain-to-source voltage, and gate-to-source voltage. A MOSFET operates in the linear ohmic region, where it behaves like a voltage-controlled resistor, and the saturation region, where the drain current becomes relatively constant. The gate-to-source threshold voltage is the minimum voltage required to establish a conducting channel between the drain and source. Through circuit simulation in LTspice, the output and transfer characteristics of a 2N7000 MOSFET are analyzed to identify these operating regions and determine its threshold voltage.

Simulations and Results:

The MOSFET circuit shown in Figure 1 was constructed and simulated in LTspice, as shown in Figure 2. Two simulations were performed to examine the MOSFET's output and transfer characteristics.

Figure #1: MOSFET Circuit Schematic (Lab Manual)

Figure #2: MOSFET Circuit (LTSpice)

For Part 1, V2 was set to 2.5 V, and V1 was increased from 0 V while measuring VGS, VDS, and ID. The process was repeated with V2 set to 3 V, 3.5 V, and 4 V. The results were recorded in Table 1 and plotted to show the relationship between drain current and drain-to-source voltage for each gate-to-source voltage. The plot in Figure 3 was used to identify the linear ohmic region, where the MOSFET behaves like a voltage-controlled resistor. The minimum VDS required for saturation at each VGS was also recorded based on the values in Figure 3.

| V2 = 2.5 V |  | V2 = 3 V |  | V2 = 3.5 V |  | V2 = 4 V |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VDS | VGS = 1.876 V | VDS | VGS = 1.958 V | VDS | VGS = 2.027 V | VDS | VGS = 2.089 V |
| (mV) | ID (mA) | (mV) | ID (mA) | (mV) | ID (mA) | (mV) | ID (mA) |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5.956 | 0.903 | 5.785 | 1.296 | 3.493 | 1.036 | 4.949 | 1.727 |
| 13.316 | 1.806 | 33.984 | 5.358 | 38.428 | 7.678 | 30.568 | 8.136 |
| 22.760 | 2.707 | 62.184 | 7.421 | 73.362 | 10.758 | 64.192 | 12.666 |
| 35.549 | 3.604 | 90.383 | 8.593 | 149.054 | 13.468 | 101.019 | 15.290 |
| 54.334 | 4.496 | 137.383 | 9.631 | 189.811 | 14.077 | 152.256 | 17.194 |
| 86.479 | 5.376 | 165.182 | 9.872 | 207.278 | 14.252 | 198.690 | 18.104 |
| 115.481 | 5.804 | 184.382 | 10.122 | 230.568 | 14.428 | 233.916 | 18.523 |
| 181.184 | 6.194 | 259.581 | 10.392 | 259.680 | 14.577 | 297.962 | 18.940 |
| 632.641 | 6.243 | 334.780 | 10.421 | 376.128 | 14.721 | 344.396 | 19.066 |
| 1132.641 | 6.243 | 438.178 | 10.421 | 498.399 | 14.727 | 382.824 | 19.111 |

Table #1: MOSFET Output Characteristics

Figure #3: ID vs. VDS at

For Part 2, V1 was set to 22 V, and V2 was varied from 0 V in 0.25 V increments until the drain current stopped increasing. The corresponding VGS and ID values were recorded in Table 2 and plotted in Figure 4 to examine the MOSFET's transfer characteristics. The threshold voltage was identified from Figure 4 as the minimum gate-to-source voltage required for the MOSFET to begin conducting.

| VGS (V) | ID (mA) |
| --- | --- |
| 0 | 0.022 |
| 1 | 0.022 |
| 1.5 | 0.022 |
| 1.75 | 2.018 |
| 2 | 12.960 |
| 2.25 | 19.766 |
| 2.5 | 19.876 |
| 3 | 19.910 |

Table #2: MOSFET Output Characteristics

Figure #3: ID vs. VDS