EE 316

Section 3

Pre Lab 12

By: Hannah Kelley

MOSFET Amplification (with Bonus)

Introduction:

This lab investigates the frequency response of a MOSFET amplifier using the 2N7000 transistor. By measuring voltage gain over a range of input frequencies, the amplifier’s low and high cutoff frequencies and approximate bandwidth can be determined. The experiment also examines the phase relationship between the input and output signals to demonstrate the phase inversion characteristic of a common-source MOSFET amplifier.

Simulations and Results:

The MOSFET amplifier circuit in Figure 1 was built and simulated in LTSpice using a 200 mV peak-to-peak input signal with frequencies varying from 10 Hz to 3 MHz. Figure 2 shows the simulated circuit. Figure 3 shows the Bode plot for the simulated circuit. Table 1 shows the high and low frequency cutoffs and the bandwidth, as seen in the Bode plot. Table 2 shows select frequencies and their Vout and gain values.

Figure #1: MOSFET Amplifier Schematic (Lab Manual)

Figure #2: MOSFET Amplifier Circuit (LTSpice)

Figure #3: MOSFET Amplifier Bode Plot

|  | Value |
| --- | --- |
| Low Frequency Cutoff | 28.8 Hz |
| High Frequency Cutoff | 33.7 kHz |
| Bandwidth | 33.67 kHz |

Table #1: Important Frequencies (Simulation)

| Frequency | Vout (V) | Voltage Gain (dB) |
| --- | --- | --- |
| 10 Hz | 0.17962 | 5.0926176 |
| 30 Hz | 0.398118 | 12.00017 |
| 60 Hz | 0.492983 | 13.861368 |
| 100 Hz | 0.523846 | 14.386605 |
| 200 Hz | 0.539604 | 14.631384 |
| 500 Hz | 0.543999 | 14.702174 |
| 1 kHz | 0.54444 | 14.709211 |
| 2 kHz | 0.543813 | 14.699192 |
| 5 kHz | 0.538357 | 14.611703 |
| 10 kHz | 0.519996 | 14.312978 |
| 15 kHz | 0.491868 | 13.841700 |
| 20 kHz | 0.461887 | 13.293075 |
| 50 kHz | 0.293064 | 9.3413204 |
| 75 kHz | 0.211748 | 6.5162771 |
| 100 kHz | 0.166314 | 4.4046672 |
| 150 kHz | 0.111264 | 1.0385469 |
| 200 kHz | 0.086358 | -1.2817469 |
| 500 kHz | 0.03472 | -9.1898298 |
| 750 kHz | 0.022947 | -12.781244 |
| 1 MHz | 0.017427 | -15.179768 |
| 1.5 MHz | 0.011491 | -18.782752 |
| 2.0 MHz | 0.008726 | -21.189108 |
| 3.0 MHz | 0.005782 | -24.752158 |

Table #2: Simulation Results

BONUS Simulation and Results:

The MOSFET amplifier circuit in Figure 1 was modified and simulated in LTspice to maximize its bandwidth while maintaining a passband gain above 10 dB. To achieve this, R1 was reduced to 1 kΩ to extend high-frequency response, and capacitors C1, C2, and C3 were set to 110 µF, 470 µF, and 100 µF respectively to push the low-frequency limit downward. To prevent output waveform clipping, supply voltage V1 was increased to 20 V, and biasing resistors R3 and R2 were adjusted to 150 kΩ and 60 kΩ to center the DC operating point. Figure 4 shows the modified circuit in LTSpice. Figure 5 displays the resulting Bode plot. Table 3 summarizes the high and low cutoff frequencies along with the total bandwidth obtained from the simulation.

Additionally, transient analysis confirmed that the output signal remained clean and unclipped across the entire passband with a 200 mV peak-to-peak input signal. Figure 6 shows a plot of Vout during this transient analysis.

The modified circuit achieved a mid-band gain of 28.6 dB, easily exceeding the minimum 10 dB requirement. The lower cutoff frequency flow was reduced to 12.3 Hz, while the upper cutoff frequency fhigh increased significantly from 33.7 kHz to 515 kHz. Overall, this expanded the amplifier's total bandwidth from 33.67 kHz to 515 kHz, roughly a 15 times improvement over the original design.

This performance increase come down to how the components handle signal frequencies. Increasing the coupling and bypass capacitors allows low-frequency signals to pass straight through to the output instead of getting blocked, which pushes the lower cutoff frequency down to 12.3 Hz. At the same time, lowering input resistor R1 from 50 kΩ to 1 kΩ removes a major bottleneck at the input gate. In the original circuit, a large R1 reacted with the transistor's internal capacitance to choke off high-frequency signals. Shrinking R1 lets those fast-changing signals pass freely, stretching the high-frequency cutoff out to 515 kHz. Furthermore, raising V1 to 20 V provided the necessary DC headroom to support the higher mid-band gain without saturating or clipping the output waveform.

Figure #4: Modified MOSFET Amplifier Circuit (LTSpice)

Figure #5: Modified MOSFET Amplifier Bode Plot

|  | Value |
| --- | --- |
| Low Frequency Cutoff | <10 Hz |
| High Frequency Cutoff | 2.83 MHz |
| Bandwidth | 2.83 MHz |

Table #3: Important Frequencies for Modified Circuit (Simulation)

Figure #6: Modified MOSFET Transient Output Plot