EE 316

Section 3

Pre Lab 09

By: Hannah Kelley

Operating Characteristics of JFETs

Introduction:

This lab introduces the Junction Field-Effect Transistor (JFET), covering n-channel and p-channel constructions, channel symmetry, and key operational parameters such as pinch-off voltage and saturation current. By examining its voltage-controlled behavior, high input impedance, and low output noise, the experiment demonstrates why JFETs excel in small-signal amplification compared to BJTs. Ultimately, it teaches how to analyze JFET biasing and operating states, establishing the foundational concepts required to understand MOSFETs in future labs.

Simulations and Results:

The JFET circuit in Figure 1 was built and simulated in LTSpice to obtain its drain and transfer characteristic curves. Using the simulated circuit in Figure 2, ID was measured across various VDS values while maintaining VGS at 0 V, -0.5 V, and -1 V by adjusting V1 and V2 independently. These ID values are summarized in Table 1. The resulting data was plotted to create the drain characteristic curve in Figure 3, from which VP and VTH were estimated and annotated. Next, with VDS fixed at 6 V, VGS was incrementally decreased until ID reached 0 mA to generate the transfer characteristic curve in Figure 4. Finally, VP was estimated from the transfer plot and compared against the drain characteristic value.

Figure #1: JFET Circuit (Lab Manual)

Figure #2: JFET Circuit (LTSpice)

Figure #3: JFET Drain Characteristic Curves

VP = 3.8 V

VTH = -2 V

|  | VGS = 0 V | VGS = -0.5 V | VGS = -1 V |
| --- | --- | --- | --- |
| VDS (V) | ID (mA) | ID (mA) | ID (mA) |
| 0 | 0 | 0 | 0 |
| 0.5 | 0.19598597 | 0.18313265 | 0.16173965 |
| 1 | 0.38969575 | 0.36175252 | 0.31208465 |
| 2 | 0.76892186 | 0.70063752 | 0.54517260 |
| 4 | 1.4777967 | 1.1808369 | 0.57418324 |
| 8 | 2.0346143 | 1.2054729 | 0.58616768 |
| 12 | 2.076169 | 1.2301051 | 0.59815182 |
| 16 | 2.1177211 | 1.2547361 | 0.61013561 |
| 20 | 2.1592702 | 1.279366 | 0.62211906 |

Table #1: Drain Characteristics

|  | VDS = 6 V |
| --- | --- |
| VGS (V) | ID (mA) |
| 0 | 2.0565675 |
| -0.5 | 1.2080352 |
| SS-1 | 0.58367301 |
| -1.5 | 0.18379232 |
| -2 | 0.008701697 |

Table #2: Transfer Characteristics

Figure #4: Transfer Characteristic Plot

VP = 3.8 V

VTH = -2 V