EE 316

Section 3

Pre Lab 08

By: Hannah Kelley

Characteristics of BJT and Amplification Behavior

Introduction:

The purpose of this laboratory is to investigate the characteristics and performance of a small-signal bipolar junction transistor (BJT) amplifier. Using LTSpice, the circuit is simulated to analyze its frequency response, output voltage, and gain. Transient analysis is also performed to identify output clipping at higher frequencies and evaluate circuit modifications intended to reduce distortion.

Simulations and Results:

The small signal BJT amplifier in Figure 1 was built and simulated in LTSpice. Figure 2 shows the BJT amplifier in LTSpice. A 500 mV peak-to-peak signal was applied to the circuit over a range of frequencies from 0.1Hz to 1MHz. The resulting Bode plot is shown in Figure 3. A selection of the output voltage and gain for a subset of frequencies is recorded in Table 1.

Figure #1: BJT Amplifier (Lab Manual)

Figure #2: BJT Amplifier Circuit (LTSpice)

Figure #3: BJT Amplifier Bode Plot

| Frequency | Vout (V) | Gain (dB) |
| --- | --- | --- |
| 10 Hz | 0.00064 | -63.8983 |
| 30 Hz | 0.00192 | -54.3559 |
| 60 Hz | 0.00383 | -48.3353 |
| 100 Hz | 0.00638 | -43.8983 |
| 200 Hz | 0.01277 | -37.8777 |
| 500 Hz | 0.03192 | -29.9189 |
| 1 kHz | 0.06384 | -23.8984 |
| 2kHz | 0.12767 | -17.8781 |
| 5 kHz | 0.31911 | -9.92114 |
| 10 kHz | 0.63773 | -3.90721 |
| 15 kHz | 0.95538 | -0.396496 |
| 20 kHz | 1.27157 | 2.08679 |
| 50 kHz | 3.11309 | 9.86382 |
| 75 kHz | 4.53336 | 13.1284 |
| 100 kHz | 5.81514 | 15.2912 |
| 150 kHz | 7.92018 | 17.9747 |
| 200 kHz | 9.46204 | 19.5197 |
| 500 kHz | 12.89258 | 22.2068 |
| 750 kHz | 13.51964 | 22.6193 |
| 1 MHz | 13.76179 | 22.7735 |
| 1.5 MHz | 13.94232 | 22.8867 |
| 2 MHz | 14.00683 | 22.9268 |

Table #1: Simulation Output Voltage and Gain

Figures 4 and 5 show the transient output of the circuit at 10 Hz and 20 MHz respectively. Figure 5 shows significant clipping as the output waveform reaches 16 Vpk-pk. After running the simulation at a variety of frequencies, the clipping seems to first occur at around 400 kHz, as shown in Figure 6.

Figure #4: Transient Output at 10 Hz

Figure #5: Transient Output at 2 MHz

Figure #6: Transient Output at 400 kHz

To reduce this clipping, RB was halved and C1 was reduced by 0.01 uF. The modified circuit is shown in Figure 7. The transient output of the modified circuit at 2 MHz is shown in Figure 8.

Figure #7: Modified BJT Amplifier Circuit (LTSpice)

Figure #8: Modified Circuit Transient Output at 2 MHz