EE 316

Section 3

Lab 08

By: Hannah Kelley

Characteristics of BJT and Amplification Behavior

Introduction:

The purpose of this laboratory is to investigate the characteristics and performance of a small-signal bipolar junction transistor (BJT) amplifier. Using LTSpice, the circuit is simulated to analyze its frequency response, output voltage, and gain. Transient analysis is also performed to identify output clipping at higher frequencies and evaluate circuit modifications intended to reduce distortion.

Simulations and Results:

The small signal BJT amplifier in Figure 1 was built and simulated in LTSpice. Figure 2 shows the BJT amplifier in LTSpice. A 500 mV peak-to-peak signal was applied to the circuit over a range of frequencies from 0.1Hz to 1MHz. The resulting Bode plot is shown in Figure 3. A selection of the output voltage and gain for a subset of frequencies is recorded in Table 1.

Figure #1: BJT Amplifier (Lab Manual)

Figure #2: BJT Amplifier Circuit (LTSpice)

Figure #3: BJT Amplifier Bode Plot

| Frequency | Vout (V) | Gain (dB) |
| --- | --- | --- |
| 10 Hz | 0.00064 | -63.8983 |
| 30 Hz | 0.00192 | -54.3559 |
| 60 Hz | 0.00383 | -48.3353 |
| 100 Hz | 0.00638 | -43.8983 |
| 200 Hz | 0.01277 | -37.8777 |
| 500 Hz | 0.03192 | -29.9189 |
| 1 kHz | 0.06384 | -23.8984 |
| 2kHz | 0.12767 | -17.8781 |
| 5 kHz | 0.31911 | -9.92114 |
| 10 kHz | 0.63773 | -3.90721 |
| 15 kHz | 0.95538 | -0.396496 |
| 20 kHz | 1.27157 | 2.08679 |
| 50 kHz | 3.11309 | 9.86382 |
| 75 kHz | 4.53336 | 13.1284 |
| 100 kHz | 5.81514 | 15.2912 |
| 150 kHz | 7.92018 | 17.9747 |
| 200 kHz | 9.46204 | 19.5197 |
| 500 kHz | 12.89258 | 22.2068 |
| 750 kHz | 13.51964 | 22.6193 |
| 1 MHz | 13.76179 | 22.7735 |
| 1.5 MHz | 13.94232 | 22.8867 |
| 2 MHz | 14.00683 | 22.9268 |

Table #1: Simulation Output Voltage and Gain

Figures 4 and 5 show the transient output of the circuit at 10 Hz and 20 MHz respectively. Figure 5 shows significant clipping as the output waveform reaches 16 Vpk-pk. After running the simulation at a variety of frequencies, the clipping seems to first occur at around 400 kHz, as shown in Figure 6.

Figure #4: Transient Output at 10 Hz

Figure #5: Transient Output at 2 MHz

Figure #6: Transient Output at 400 kHz

To reduce this clipping, RB was halved and C1 was reduced by 0.01 uF. The modified circuit is shown in Figure 7. The transient output of the modified circuit at 2 MHz is shown in Figure 8.

Figure #7: Modified BJT Amplifier Circuit (LTSpice)

Figure #8: Modified Circuit Transient Output at 2 MHz

Experimental Setup and Results:

The BJT amplifier in Figure 1 was built using a breadboard, resistors, capacitors, and a BJT. Figure 9 shows a picture of the experimental setup. WaveForms was used to record the Bode plot in Figure 10. In Figure 10, the small bumps present at low frequencies are likely caused by the capacitors affecting the signal differently as the frequency changes. They could also be minor simulation irregularities. Table 2 summarizes specific points on the Bode plot and voltage values at varying frequencies.

Figure #9: BJT Amplifier Experimental Setup

Figure #10: BJT Amplifier Bode Plot

| Frequency | Vout (V) | Gain (dB) |
| --- | --- | --- |
| 10 Hz | 0.000103 | -73.723 |
| 30 Hz | 0.000216 | -67.295 |
| 60 Hz | 0.000976 | -54.193 |
| 100 Hz | 0.000675 | -57.394 |
| 200 Hz | 0.001641 | -49.677 |
| 500 Hz | 0.006919 | -37.179 |
| 1 kHz | 0.021636 | -27.276 |
| 2kHz | 0.050422 | -19.927 |
| 5 kHz | 0.13419 | -11.425 |
| 10 kHz | 0.282957 | -4.945 |
| 15 kHz | 0.425618 | -1.399 |
| 20 kHz | 0.589006 | 1.423 |
| 50 kHz | 1.417816 | 9.053 |
| 75 kHz | 1.622632 | 10.225 |
| 100 kHz | 1.680041 | 10.527 |
| 150 kHz | 1.708126 | 10.671 |
| 200 kHz | 1.694416 | 10.601 |
| 500 kHz | 1.663106 | 10.439 |
| 750 kHz | 1.662149 | 10.434 |
| 1 MHz | 1.664639 | 10.447 |
| 1.5 MHz | 1.669053 | 10.470 |
| 2 MHz | 1.670782 | 10.479 |

Table #2: Experimental Voltage and Gain

Discussion:

The simulation and experimental results matched somewhat closely. The simulation Bode plot was overall much smoother, though the general shape of the simulation and experimental Bode plots matched. Low frequency noise was present in the experimental Bode plot, likely caused by the capacitors affecting the signal differently as the frequency changes. They could also be minor simulation irregularities. Table 3 shows the percent errors for Vout and gain. The percent error for Vout are all very large, likely due to the voltage values being in the millivolt range. In actuality, the simulation and experimental voltage values were very close. The percent errors for gain were also generally close with larger variations at higher frequencies. The large percent error at 15 kHz was caused by a simulation value of -0.3 dB and an experimental value of -1.399 dB, making the small deviation seem much larger than it actually is.

| Frequency | Vout | Gain |
| --- | --- | --- |
| 10 Hz | 83.91% | 15.38% |
| 30 Hz | 88.76% | 23.80% |
| 60 Hz | 74.53% | 12.12% |
| 100 Hz | 89.42% | 30.74% |
| 200 Hz | 87.15% | 31.15% |
| 500 Hz | 78.33% | 24.27% |
| 1 kHz | 66.11% | 14.13% |
| 2kHz | 60.51% | 11.46% |
| 5 kHz | 57.95% | 15.16% |
| 10 kHz | 55.63% | 26.55% |
| 15 kHz | 55.45% | 252.72% |
| 20 kHz | 53.68% | 31.81% |
| 50 kHz | 54.46% | 8.22% |
| 75 kHz | 64.21% | 22.12% |
| 100 kHz | 71.11% | 31.16% |
| 150 kHz | 78.43% | 40.64% |
| 200 kHz | 82.09% | 45.69% |
| 500 kHz | 87.10% | 53.00% |
| 750 kHz | 87.71% | 53.87% |
| 1 MHz | 87.90% | 54.12% |
| 1.5 MHz | 88.03% | 54.20% |
| 2 MHz | 88.07% | 54.28% |

Table #3: Voltage and Gain Percent Errors

Conclusion:

In conclusion, this lab investigated the characteristics and performance of a small-signal BJT amplifier using LTSpice. Frequency response, output voltage, and gain were analyzed through simulation, while transient analysis was used to observe clipping and distortion at higher frequencies. The simulated results were compared with experimental measurements, revealing differences likely caused by component tolerances, parasitic effects, and measurement limitations. Overall, the lab demonstrated how frequency and circuit characteristics affect amplifier performance and how circuit modifications can help reduce distortion.